Translation from Romanian

HISTORICAL STUDY FOR THE ANALYSIS OF THE POSSIBILITIES OF FUNCTIONAL CONVERSION OF THE ANNEXES, THE GREENHOUSE AND THE ENHANCEMENT OF THE LIBRECHT - FILIPESCU HOUSE - today the University House (historical monument - code LMI B-II-m-A-19107)

STRADA DIONISIE LUPU, NR. 46,

SECTOR 2, MUNICIPIUL BUCURESTI

SHEET NO. 1.3. - HISTORICAL EVOLUTION OF THE STREET NETWORK, THE PLOT AND THE BUILT FACILITY - COMPARATIVE ANALYSIS OF THE YEARS 1911-1974

| COMPARATIVE ANALYSIS OF THE STREET NETWORK 1911-1974 |  | COMPARATIVE ANALYSIS OF PLOT NETWORK 1911-1974 |  | COMPARATIVE ANALYSIS OF BUILT-UP FUND 1846-1911 |  |
| --- | --- | --- | --- | --- | --- |
| 1911 - MAP OF THE ARMY INSTITUTE | 1974 - BUCHAREST - HISTORIC CENTER | 1911 - MAP OF THE ARMY INSTITUTE | 1974 - BUCHAREST - HISTORIC CENTER | 1911 - MAP OF THE ARMY INSTITUTE | 1974 - BUCHAREST - HISTORIC CENTER |
| Contemporary limit of the plot related to the University House Study area limit Existing circulation arteries at the date of the plan, included in the study area |  | Contemporary limit of the plot related to the University House Study area limit The limits of the plots at the date of the plan, included within the limits of the study area |  | Contemporary limit of the plot related to the University House Study area limit Existing constructions at the date of the plan, within the limits of the study area Constructions that make up the University House complex |  |
| Contemporary limit of the plot related to the University House Study area limit Traffic arteries that appeared before 1911 and were maintained between 1911 and 1974 Areas expropriated for the purpose of modifying the street network Areas returned to private property following street alignment rectification Public green spaces created before 1911 and maintained between 1911 and 1974 The street network did not undergo major changes during this period, maintaining the arteries developed prior to 1946 and during the period 1846-1911. However, minor interventions are being undertaken to rectify the alignment, resulting in areas returned to private property, respectively expropriated areas. All traffic arteries in the study area receive similar street prospects and an average importance at the municipal level. The plot of the University House has access to three streets: Dionisie Lupu (to the west) Pictor Arthur Verona (to the north) and Alexandru Sahia (to the east). |  | Contemporary limit of the plot related to the University House Study area limit Parcel limits at the time of the creation of the Army Geography Institute Map, 1911 Parcel system that appeared before 1911 and was maintained during the period 1911 -1974 Plots created between 1911-1974 through division Plots created between 1911-1974 by annexation Areas where the density of the parcels increases during the period 1911-1974 following the division actions The morpho-typological characteristics of the structure do not undergo major changes during this period, maintaining the elongated parcels, with great depth compared to the opening to the street. However, areas are observed where the density of the parcels increases following the division interventions. Atypical parcels are also maintained, with very large surfaces. as is the case of the land on which the Librecht-Filipescu House is located, which became the University House during this period (after 1947). With an area of ​​14,555 sq m. the corner plot opens onto three streets, occupying the entire northern front of the island. |  | Contemporary limit of the plot related to the University House Study area limit — Constructions that appeared before 1911 Constructions that appeared between 1911 and 1974 Areas that underwent a significant densification of the footprint of the built fund between 1911 and 1974 Constructions classified as a historical monument, by the List of Historical Monuments 2015, within the perimeter of the studied area In the first half of the 20th century, a densification of the built surface took place in the southern part of the studied area. The presence of a planted public space and seven buildings currently classified as a historical monument, according to L.M.I. 2015 code, is also observed. During the studied period, a densification of the annexed construction fund took place, especially on the southern side of the plot. Most of these no longer exist or are in an advanced state of degradation. Those that are maintained are mainly the constructions made between 1846-1911. The greenhouses are also extended, by adding an additional building to the north. |  |

Most of the modifications that take place during this period are made in the first half of the 20th century, in the period between the two world wars. During this period, the tendency to sequence the plot is noticeable, through the way the built stock is organized and its relationship to the plot and its accesses.

In 1974, when the plan was drawn up, three sequences can be identified: 1. Librecht-Filipescu House, with the related annexes and the main access to the plot, from Dionisie Lupu Street: 2. The park, which follows the House and we call it that because the plan shows a landscape design with alleys and planted massifs; 3. The greenhouse, in the eastern third of the plot, towards Alexandru Sahia Street (Jean Louis Calderon Street), where there is also a landscaping, but less complex than that in the middle part of the land.

In the first half of the 20th century, between 1911 and 1974, three other buildings appeared that were classified as historical monuments starting in 1992 and are currently listed in the 2015 List of Historical Monuments:

1 Central School for Girls (L.M.I. 2015 code: B-II-m-A-18924), 1890 - Icoanei street, no. 3-5

2 Icoanei Garden (L.M.I. 2015 code: B-II-a-B-18301), late 19th century - early 20th century - Gheorghe Cantacuzino Square, without house number

3 Librecht-Filipescu House – Today, University House (L.M.I. 2015 code: B-II-m-A-19107), mid-19th century - Dionisie Lupu Street, no. 46

4 Prof. Dr. Turnescu House (L.M.I. 2015 code: B-II-m-B-19106) - end of the 19th century - Dionisie Lupu Street. no. 37

5 House (L.M.I. 2015 code: B-II-m-B-19624), end of the 19th century - C.A. Rosetti, nr. 33

6 House (L.M.I. 2015 code: B-II-m-B-19625), end of the 19th century - C.A. Rosetti, nr. 35

7 Church of the "Resurrection" - Anglican (L.M.I. 2015 code: B-II-m-B-19833) - early 20th century Pictor Arthur Verona street, no. 13

8 Gh. Gr. Cantacuzino Monument (cod L.M.I. 2015 code: B-III-m-B-19968) - Gheorghe Cantacuzino Square, without house number, in the Icoanei Garden

| DESIGNER: S.C. LOGIC'PLAN S.R.L. J40/11100/22017, Sole Registration Code (CUI): 37883598 | SIGNATURE: |  | JECT MANAGER Laura Elena TUCAN Urban planner, RUR member Specialist certified by the Ministry of Culture | SIGNATURE: | ARCHITECTURE COORDINATOR: George - Bogdan TEODORESCU Architect, OAR Member, Specialist certified by the Ministry of Culture |  | SIGNATURE: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PROJECT NAME: |  |  |  | DESIGNED/ELABORATED: |  | SIGNATURES: |  |
| Historical study for the analysis of the possibilities of functional conversion of the annexes, the greenhouse and the courtyard of the Librecht - Filipescu House - today the University House (LMI B-II-m-A-19107) |  |  |  | Urban planner Minerva CIOIU |  |  |  |
|  |  |  |  | Urban planner Tania DOBRE |  |  |  |
|  |  |  |  | Urban planner Antonia PANAITESCU |  |  |  |
|  |  |  |  | Urban planner Elena - Cristina PELMUS |  |  |  |
| SHEET NAME: |  | HISTORICAL EVOLUTION OF THE STREET NETWORK, THE PLOT AND THE BUILT FACILITY - COMPARATIVE ANALYSIS OF THE YEARS 1911-1974 |  | SCALE: - |  | PROJECT NO.: 6407/09306.2023 |  |
|  |  |  |  | ELABORATION DATE: |  | JULY 2023 |  |
| SHEET NUMBER: |  | SHEET 1.3. |  | BENEFICIARY: |  | BUCHAREST UNIVERSITY |  |