Translation from Romanian

HISTORICAL STUDY FOR THE ANALYSIS OF THE POSSIBILITIES OF FUNCTIONAL CONVERSION OF THE ANNEXES, THE GREENHOUSE AND THE ENHANCEMENT OF THE LIBRECHT - FILIPESCU HOUSE - today the University House (historical monument - code LMI B-II-m-A-19107)

STRADA DIONISIE LUPU, NR. 46,

SECTOR 2, MUNICIPIUL BUCURESTI

SHEET NO. 1.4. - HISTORICAL EVOLUTION OF THE STREET NETWORK, THE PLOT AND THE BUILT FACILITY - COMPARATIVE ANALYSIS OF THE YEARS 1974-2023

| COMPARATIVE ANALYSIS OF THE STREET NETWORK 1974-2023 |  | COMPARATIVE ANALYSIS OF PLOT NETWORK 1974-2023 |  | COMPARATIVE ANALYSIS OF BUILT-UP FUND 1974-2023 |  |
| --- | --- | --- | --- | --- | --- |
| 1974 - BUCHAREST - HISTORIC CENTER | 2023 - ORTHOPHOTOMAP | 1974 - BUCHAREST - HISTORIC CENTER | 2023 - ORTHOPHOTOMAP | 1974 - BUCHAREST - HISTORIC CENTER | 2023 - ORTHOPHOTOMAP |
| The limit of the plot related to the University House Study area limit Existing circulation arteries at the date of the plan, included in the study area |  | The limit of the plot related to the University House Study area limit The limit of the plots on the planning date included within the study area |  | Contemporary limit of the plot related to the University House Study area limit Existing constructions at the date of the plan, within the limits of the study area Constructions that comprise the University House complex |  |
| Contemporary limit of the plot related to the University House Study area limit Traffic arteries that appeared before 1974 and were maintained during the period 1974-2023 Green spaces created before 1974 and maintained during the period 1974-2023 The pedestrian public square that appeared between 1974-2023 The six historical traffic arteries are currently maintained, having approximately similar importance. Four of these are collector streets, category III, facilitating connections in the west-east direction (G-ral. Gh. Magheru Boulevard - Calea Mosilor) and in the north-south direction (Stefan cel Mare Road - Universitatii Square), while the other two, Pictor Arthur Verona and A. D. Xenopol streets, are local streets, category IV. The plot on which the University House is located has access to three streets: Dionisie Lupu, Pictor Arthur Verona and Jean Louis Calderon. Of these, Pictor Arthur Verona has the least importance at the level of the municipal street network, but it represents the connecting space between the Icoanei House and Garden. |  | Contemporary limit of the plot related to the University House Study area limit The limit of the plots at the time of the Bucharest-Historic Center plan, 1974 Parcel structure that appeared before 1974 and was maintained during the period 1974-2023 Parcel list appeared between 1974-2023 ((The imperfect overlapping of the plots comes from the differences in scale and graphic representation of the two plans) The parcel layout does not undergo significant changes during this period, due to the high degree of protection, through the establishment of protected areas built in 1991. Urban planning operations are punctual during this period, taking place mainly after 1991. In the eastern part of the studied area, a series of cases of parcel divisions are observed, which, by generalization, may affect the historical, cultural and urban qualities of the parcel. |  | Contemporary limit of the plot related to the University House Study area limit Constructions that appeared before 1974 Constructions that appeared between 1974-2023 Disappeared buildings in the period 1974-2023 (by demolition or accident) Constructions classified as historical monuments, through the List of Historical Monuments 2015, within the perimeter of the studied area (The imperfect overlapping of the plots comes from the differences in scale and graphic representation of the two plans.) The historical built fund is maintained and well preserved during this period, with minor changes, indicated in the plan: the disappearance of some buildings and new insertions. Less represented on the ground but still existing are the facade modifications or the addition of floors. Six new buildings were added to the University House ensemble during this period: a kitchen, two repositories, an events pavilion, a greenhouse and a warehouse. The last two extend existing buildings, but all are located at the rear of the plot, being invisible from Dionisie Lupu Street. The greenhouses, on the other hand, are visible from Pictor Arthur Verona Street. right next to the access implemented between 1911-1974. |  |

Like the previous period, the 1974-2023 interval means the densification of the annexed buildings within the Librecht-Filipescu House. This is the period in which the historical monument becomes the University House, under the administration of the Bucharest University. The built annexes (outdoor kitchen, metal structure, greenhouse extension, warehouse) show the orientation of the ensemble's function towards public catering and services: The house becomes a restaurant with a terrace, and the greenhouses become a space for selling plants during the 1990s. Currently, the ensemble is mostly inactive from a functional point of view, except for occasional events.

Legend

Constructions built between 1974-2023

Constructions built before 1974

The University House is directly visible from Dionisie Lupu Street, being located with the main facade facing it.

The constructions built between 1974 and 2023 located at the rear of the University House are not visible from Dionisie Lupu Street, unlike the greenhouses that are visible from Pictor Arthur Verona Street.

In the study area there are eight historical monuments of architecture and public space, both built heritage and planted public space. These appear before the period 1974-2023 and are classified in the 2015 List of Historical Monuments as follows:

1 Central School for Girls (L.M.I. 2015 code: B-ll-m-A-18924), 1890 Icoanei street, no. 3-5

2 Icoanei Garden (L.M.I. 2015 code: B-II-a-B-18301). end of the 19th century-beginning of the 20th century - Gheorghe Cantacuzino Square, without house number

3 Librecht-Filipescu House – Today, University House (L.M.I. 2015 code: B-ll-m-A-19107), mid-19th century - Dionisie Lupu Street, no. 46

4 Prof. Dr. Turnescu House (L.M.I. 2015 code: B-II-m-B-19106) - end of the 19th century - Dionisie Lupu Street, no. 37

5 House (L.M.I. 2015 code: B-II-m-B-19624), end of the 19th century - C.A. Rosetti Street, no. 33

6 House (L.M.I. 2015 code: B-II-m-B-19625), end of the 19th century - C.A. Rosetti, nr. 35

7 Church of the "Resurrection" - Anglican (L.M.I. 2015 code: B-II-m-B-19833), early 20th century Pictor Arthur Verona street, no. 13

8 Gh. Gr. Cantacuzino Monument (cod L.M.I. 2015 code: B-III-m-B-19968) - Gheorghe Cantacuzino Square, without house number, in the Icoanei Garden

| DESIGNER: S.C. LOGIC'PLAN S.R.L. J40/11100/22017, Sole Registration Code (CUI): 37883598 | SIGNATURE: |  | JECT MANAGER Laura Elena TUCAN Urban planner, RUR member Specialist certified by the Ministry of Culture | SIGNATURE: | ARCHITECTURE COORDINATOR: George - Bogdan TEODORESCU Architect, OAR Member, Specialist certified by the Ministry of Culture |  | SIGNATURE: |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PROJECT NAME: |  |  |  | DESIGNED/ELABORATED: |  | SIGNATURES: |  |
| Historical study for the analysis of the possibilities of functional conversion of the annexes, the greenhouse and the courtyard of the Librecht - Filipescu House - today the University House (LMI B-II-m-A-19107) |  |  |  | Urban planner Minerva CIOIU |  |  |  |
|  |  |  |  | Urban planner Tania DOBRE |  |  |  |
|  |  |  |  | Urban planner Antonia PANAITESCU |  |  |  |
|  |  |  |  | Urban planner Elena - Cristina PELMUS |  |  |  |
| SHEET NAME: |  | HISTORICAL EVOLUTION OF THE STREET NETWORK, THE PLOT AND THE BUILT FACILITY - COMPARATIVE ANALYSIS OF THE YEARS 1974-2023 |  | SCALE: - |  | PROJECT NO.: 6407/09306.2023 |  |
|  |  |  |  | ELABORATION DATE: |  | JULY 2023 |  |
| SHEET NUMBER: |  | SHEET 1.4. |  | BENEFICIARY: |  | BUCHAREST UNIVERSITY |  |